To ensure VirusTC clinical staff maintain the highest standards of precision and diagnostic integrity while advancing our goal of scalpel-free medicine, please utilize the following checklist for all laser-based dental and oncological procedures.

I. Pre-Procedure: Software & Digital Mapping

- [ ] Vector Optimization: Verify that the digital surgical path is designed in CAD/Illustrator with a 0.001" stroke width.

- [ ] Shape Alignment: Align the digital path with the physical boundary projected onto the patient's tissue.

- [ ] Parameter Calibration: Set initial parameters based on tissue density:

  - Power: 4–6 Watts (standard for oral mucosa).

  - Speed: 8 mm/s.

  - Mode: Gated Pulse or Super-Pulse (to minimize thermal relaxation time).

II. Intraoperative Protocol: Tissue Manipulation

- [ ] Tension Application: For soft tissue excisions, manually spread the tissue at a 90-degree angle to Langer’s Lines.

- [ ] Stabilization: Secure the spread tissue with surgical tape to maintain constant tension during computer-guided ablation.

- [ ] The "Single Pass" Objective: Aim to complete the ablation in one pass. Avoid repeated low-power passes, which increase cumulative thermal damage.

- [ ] Orientation Marking: Place a permanent marker or suture at the 12 o'clock position before final excision to ensure accurate pathological orientation.

III. Specimen Handling & Stabilization

- [ ] Immediate Fixation: Submerge the specimen in 10% Neutral Buffered Formalin (NBF) immediately upon removal.

- [ ] Fixative Ratio: Confirm a minimum ratio of 10 parts formalin to 1 part tissue.

- [ ] Targeted Labeling: Clearly mark the container as "Laser Excision" to alert the pathology lab to potential thermal artifacts.

IV. Post-Procedure Troubleshooting (The Feedback Loop)

- [ ] Charring Inspection: Inspect the specimen for carbonization. If the "char" layer exceeds 0.5 mm, diagnostic integrity is at risk.

- [ ] Parameter Adjustment: If significant charring or "streaming" nuclei (thermal stretching) are reported by the pathologist:

  - Increase programmed speed ($mm/s$).

  - Switch to a higher pulse frequency to allow for cooling.

- [ ] Pathology Coordination: Request "Deeper Sectioning" (levels) if the primary surface is too charred for a definitive assessment.

V. Maintenance & Quality Control

- [ ] Focal Point Calibration: Re-calibrate the laser focal point weekly or if incision widths deviate from programmed specs.

- [ ] Safety Equipment: Confirm all staff and patients are wearing wavelength-specific protective eyewear before activating the laser.

Administrative Note: This protocol is a live document of VirusTC and the University of Washington Surgical Oncology series. Any deviations must be documented and reviewed by the Chief Medical Officer.